Table1a. Characteristics of Alarm Events

| Characteristic | All Alarms | Technical Alarms | Clinical Alarms |
| --- | --- | --- | --- |
| Events, n (%) | 193,852 (100%) | 134,739 (69.5%) | 59,113 (30.5%) |
| Alarm density (/bed/day) | 82.9 | 57.7 | 25.3 |
| Priority |  |  |  |
| ADVISORY, n (%) | 135,783 (70%) | 134,134 (99.6%) | 1,649 (2.8%) |
| WARNING, n (%) | 57,847 (29.8%) | 605 (0.4%) | 57,242 (96.8%) |
| CRISIS, n (%) | 222 (0.1%) | 0 (0%) | 222 (0.4%) |
| Dismissed alarms |  |  |  |
| Dismissed, n (%) | 5,759 (3%) | 914 (0.7%) | 4,845 (8.2%) |
| Time-to-dismiss, median [IQR], sec | 20 [10-49] | 206 [17-3025] | 18 [10-36] |
| Alarm duration |  |  |  |
| Duration, median [IQR], sec | 6 [1-20] | 4 [1-17] | 12 [4-23] |

The study was conducted in a surgical ward comprising 41 monitored beds (12 high-acuity, 29 general) over a 57-day observation period (July 15 – September 9, 2025). Technical alarms indicate device or signal artifacts (e.g., SpO2 probe disconnection, electrode failure, signal loss). Clinical alarms indicate physiological parameter deviations or arrhythmias. Alarm density was calculated as total events divided by the number of monitored beds and observation days. Time-to-dismiss was calculated only among dismissed alarms (silenced = TRUE). IQR, interquartile range.

Table1b. Response Characteristics of Clinical Alarms Stratified by Priority Level

| Characteristic | ADVISORY | WARNING | CRISIS |
| --- | --- | --- | --- |
| Events, n (% of clinical alarms) | 1,649 (2.8%) | 57,242 (96.8%) | 222 (0.4%) |
| Events, n (% of ALL alarms) | 1,649 (0.9%) | 57,242 (29.5%) | 222 (0.1%) |
| Dismissed, n (%) | 61 (3.7%) | 4,626 (8.1%) | 158 (71.2%) |
| Time-to-dismiss, median [IQR], sec | 19 [12-69] | 19 [10-37] | 8 [6-12] |
| Duration, median [IQR], sec | 18 [10-19] | 11 [4-23] | 31 [9-37] |
| Ward zone |  |  |  |
| High-acuity zone, n (%) | 770 (46.7%) | 8,150 (14.2%) | 105 (47.3%) |
| General zone, n (%) | 879 (53.3%) | 49,092 (85.8%) | 117 (52.7%) |

Priority levels are defined by the bedside monitoring system: ADVISORY indicates a low-urgency physiological deviation; WARNING indicates a potentially serious condition requiring attention; CRISIS indicates a life-threatening condition requiring immediate intervention. Time-to-dismiss was calculated only among dismissed alarms. The proportion of each priority level relative to all alarms (including technical) reflects the signal-to-noise ratio faced by nursing staff. Ward zone classification: high-acuity zone comprised 12 beds (rooms 1101-A–E, Re-A/B, W10-01–05); general zone comprised 29 beds. IQR, interquartile range.